# F. E. C. O.

| FECO: | INTERMEC # 1/1 |
| --- | --- |
| PRODUCT: | EasyCoder C4 Bar Code Label Printer |
| PRODUCT WEB SITE: | http://www.Intermec.com |
| RELEASE DATE: | 4th October 2004 |
| ESTIMATED MAN HOURS: | N/A |
| CLASS OF CHANGE: | New Product |
| REASON FOR CHANGE: |  |
| RELATED FECO/AB: | N/A |

# EasyCoder C4 Bar Code Label Printer

## Section	Page

1.	Note	1

2.	Introduction	2

3.	Overview	2

4.	Installation	2

5.	Test with O/S	3

6.	Optional Memory Cartridge	4

7.	Controls and Indicators	5

8.	Media Load Overview	6

9.	Label Gap Sensor	7

10.	Cleaning	8

11.	Troubleshooting	8

12.	Specifications	9

13.	Interfaces	9

# 1.	Note

Throughout this FECO a SPACE in a string is shown by the symbol  and all other spaces are purely cosmetic. A carriage return is shown either by the symbol  or <CR>

# 2.	Introduction

2.1	This FECO gives an overview and basic setup instructions for the EasyCoder C4 Bar Code Label Printer. For more detailed information see the Manuals & Software CD supplied with the printer, download the PDF’s from the EDP web site http://www.nsd.fastfreenet.com, or visit the manufacturers web site at http://www.Intermec.com

2.2	Read the Safety Instructions that are included in the user guide PDF

2.3	This printer has ONLY been evaluated with the Charisma application for printing despatch labels

2.4	This printer will NOT be maintained by EDP

# 3.	Overview

3.1	Intermec EasyCoder C4 is a series of dedicated direct thermal and combined thermal transfer/direct thermal printers with a print head density of 8 dots per mm (203.2 dots per inch)

3.2	The EasyCoder C4 printers are fitted with both a parallel Centronics port and a serial RS-232 port

3.3	All EasyCoder C4 printers are delivered with the Intermec InterDriver for Microsoft Windows 95/98/Me, and Windows NT 4.0/W2000. The InterDriver makes it possible to produce printouts from most standard programs run under the various versions of MS Windows

3.4	Label design becomes an easy task with Intermec LabelShop, a series of “what-you-see-is-what-you-get” PC label-editing programs and compatible with the full range of Intermec EasyCoder printers. Intermec LabelShop START is included in the delivery

3.5	EasyCoder C4 can also be run by means of ESim control commands described in the EasyCoder C4, Programmer’s Guide. These control commands use short lines of ASCII characters to format the labels, enter text and bar codes and control the printing, making it possible to use a terminal or personal computer, that does not operate under MS Windows

# 4.	Installation

4.1	Place the printer in a suitable location that allows easy access to printed labels and preferably also easy reload of media and transfer ribbon. The printer should only be operated while resting on a horizontal surface, like a table or similar

4.2	Place the power supply in a suitable location between the printer and an electrical outlet, for example on the floor. The power supply can be used for 100 to 240 VAC, 50 to 60 Hz

4.3	Check that the printer's On/Off switch at the back is off (O)

4.4	First, fit the round connector into the power receptacle on the printer's rear plate. Then connect the power cord between the power supply and an AC wall socket

4.5	The power can be switched on/off by means of the On/Off switch situated at the back of the printer. The indicator lamp on the printer lights green, red, or orange to indicate that the power is on. (It also indicates various conditions)

4.6	All EasyCoder C4 models are fitted with a 36pin female Centronics connector for the parallel interface port and a DB-9pin female connector for the RS-232 serial interface port. You can have cables connected to both of these ports simultaneously, but only one can be used at a time

4.7	If you intend to use the Intermec InterDriver or any version of Intermec LabelShop, connect the parallel communication cable delivered between the printer’s Centronics interface connector and the parallel port of your PC (LPT1)

The parallel interface can also be used for programming the printer by means of the Esim protocol described in the EasyCoder C4, Programmer’s Guide. However, prompts and other information from printer to host cannot be transmitted, since the parallel interface is one-way (host-to-printer) only

4.8	The serial RS-232 channel is intended for running the protocol described in the EasyCoder C4, Programmer’s Guide. It is not suited for the Intermec InterDriver driver

4.9	The RS-232 communication setup is variable by means of the Y command (see EasyCoder C4, Programmer’s Guide). Use the Test Mode to check the printer's present setup (bold letters indicate default setup)

Baud rate:	1200, 2400, 9600, or 19200

Parity:		None, Odd, or None

Data bits:	7 or 8

Stop bits:	1 or 2

Flow control:	XON/XOFF and DSR/CTS

# 5.	Test with O/S

5.1	To test the printer from the command line it must be set to dump mode: -

After having loaded the printer with full width labels, tickets, or tags, switch off the power to the printer

5.2	Press and hold the Feed key while switching on the power

Release the Feed key when the green indicator lamp flashes

The printer will automatically adjust the label gap sensor and media feed according to the characteristics of the media

The printer may feed out 3-4 forms before the adjustment is completed. In case of peel-off operation, remove the labels as they are fed out

After a delay, a Test Label will be printed and the printer will be placed in the Dump Mode

5.3	While in the dump mode you may send prints from the command line to test the connection and handshaking

5.4	Tap the Feed key once to switch back to normal operation

NOTE: Do not use continuous stock in the Test Mode. The printer will not be able to detect any gaps and an error will occur

# 6.	Optional Memory Cartridge

6.1	The EasyCoder C4 is fitted with a built-in flash memory where 200K is available to the user for storing forms, graphics, and fonts

6.2	The built-in memory can be expanded by means of an exchangeable memory cartridge with 1MB flash memory and/or 256K SRAM

6.3	Ensure that the On/Off switch on the printer is off (0)

Open the top cover by pressing the two locks on either side

Remove the cartridge slot cover located to the left of the media roll. Be careful not to drop it inside the printer

Insert the memory cartridge into the cartridge slot. Its slightly curved shape ensures that it can only be fitted correctly. Push firmly to seat the cartridge

Firmly close the top cover and switch on the power

# 7.	Controls and Indicators

7.1	The EasyCoder C4 is controlled by the operator by means of an On/Off switch, a “Feed” key and a multi-color indicator lamp

7.2	On/Off Switch

The On/Off switch is located at the back of the printer

7.3	Indicator Lamp

When the power to the printer is switched on, the indicator lamp will glow green if the printer is loaded with media and – in case of thermal transfer printing – ribbon

If the printer is out of media and/or ribbon, the indicator lamp will glow red

Any error condition, such as label stop sensor problems, media jams, or software errors, will be indicated by the lamp changing to orange

If the indicator lamp fails to work as described above, please refer to Section 10

7.4	Feed Key

The Feed key can be used in two ways:

- Tapping

- Holding

When power is first applied and the printer is loaded with media (and possibly ribbon), tapping the Feed key will initiate a form feed, that is media will be fed out to the top of next label or tag

When power is first applied with no labels loaded, holding the Feed key will cause the media to be continuously fed out until the key is released. This mode is useful when replenishing the media supply

After the media has been threaded through the printer, tapping the Feed key 3 more times (for a total of 4 taps) will cause a form feed

# 8.	Media Load Overview

8.1	The EasyCoder C4 can print on heat-sensitive direct thermal paper, and in case of the thermal transfer model also on non heat-sensitive receiving face materials, in the form of self-adhesive labels, non-adhesive tags, or continuous stock. In case of non heat-sensitive face materials, a suitable thermal transfer ribbon must be used

The media stock can be accommodated inside the printer in the form of a roll, or be placed behind the printer and inserted through a slot in the cover (for example fan-folded tickets or tags)

Three types of operation are available, Tear-Off (Straight through), Peel-Off (Self-Strip), and Cut-Off

WARNING	WARNING	WARNING	WARNING

If the printer runs out of media while printing, do not switch off the power during reload, or data will be lost!

8.2	Tear-Off

Tear-off (Straight-through) means that the media is fed straight out from the front of the printer and can be torn-off manually by pulling it upwards against the edge of the printhead.

Tear-off mode can for example be used for tearing off tags at the perforation, or tearing off the liner (backing paper) between labels

8.3	Peel-Off

Peel-off (Self-strip) means that self-adhesive labels are automatically separated from the liner (backing paper) after printing. It cannot be used in connection with a paper cutter.

The labels are fed straight out from the printer, whereas the liner is fed out separately from a slot further down on the printer's front, from where it can be lead to for example a wastebasket. Be careful when loading self-adhesive labels. If labels are not flat on the liner, the exposed edges can stick to your printer and cause problems

8.4	Cut-Off

Cut-off requires an optional paper cutter fitted in place of a peel-off device and means that the liner of self-adhesive labels is automatically cut between labels or non-adhesive continuous strip is cut into tickets and tags. The cutter is not designed to cut through any adhesive, which would stick to the cutting shears and prevent the cutter from operating

8.5	The built-in Label Taken Sensor (LTS) can be enabled to hold the printing until the previous label (or similar) has been removed from the printer's outfeed slot. As long as the sensor detects a label, the printer will be BUSY and cannot receive data from host

8.6	If using the ESim protocol described in the ESIM v3.33, Programmer’s Reference Manual, note the following:

- The LTS can be enabled/disabled by an O command

- When switching between peel-off, tear-off operation and cutoff operation, the media feed must be readjusted by means of a j command

8.7	The Intermec InterDriver provides options for selecting the following modes of operation without any j or O commands having to be entered by the operator:

- Tear-off

- Peel-off with LTS enabled

- Peel-off with LTS disabled

8.8	Refer to the user guide.PDF for instructions on how to load the various media and ribbon

# 9.	Label Gap Sensor

8.1	The EasyCoder C4 printer is fitted with a label gap sensor that detects slots between tickets and tags, or gaps between labels, as the media is fed past the sensor during printing. Thus the firmware can determine the length of the tags or labels and control the media/ribbon feed accordingly

9.2	The label gap sensor is a center-aligned photoelectric sensor that measures the light that passes through the media path. The transparency of the liner of a label supply may differ between batches, making it difficult for the sensor to discriminate between labels and liner. When this occurs, the indicator lamp will switch from green to orange, indicating that sensor should be adjusted by entering the Test Mode, as described the user guide PDF

# 10.	Cleaning

10.1	Make sure that the customer is aware of the following. (For more details refer to the user guide PDF)

10.2	External Cleaning

Keep your EasyCoder C4 clean by periodically wiping it with a soft cloth dampened with water. Do not use abrasive cleaners or solvents, as they will scratch the surface

10.3	Printhead Cleaning

Intermec recommend using the special Cleaning Card (part number 1-110501-00) and the procedures below to clean the printhead before loading each new roll of labels or tags

1. Switch off the power to the printer

2. Open the top cover/print carriage

3. Unload the media stock

4. In case of thermal transfer printing, also remove the transfer ribbon

5. Open the pouch and pull out the cleaning card. Be careful not to tear the card inside!

6. Insert the cleaning card under the print carriage the same way as when loading the media. Allow approximately 2 to 3 cm (1 inch) of the cleaning card to extend in front of the printhead

7. Fold down the print carriage to locked position

8. Use one hand to hold the printer and the other hand to pull the cleaning card forward, until the entire card has been pulled free

9. Repeat steps 6, 7, and 8 a second time, and then proceed at step 10

10. Dispose properly of the used cleaning card and reload the media and ribbon supplies

# 11.	Troubleshooting

| Problem | Solution or Reason |
| --- | --- |
| Power indicator does not light green when power is on | - Make sure connectors on power supply are securely plugged into the receptacle on the printer and to an AC outlet |
| Power indicator lights green but printer will not feed | - Label taken sensor active, label not removed - Make sure correct type of interface cable is securely plugged into both printer and computer |
| Printer seems to be working but nothing is printed | - Direct Thermal Printing: Verify that the media is intended for direct thermal printing by testing if the paper is blackened by the heat from a hot object (+ 70°C/160°F or more). Check that the heat-sensitive side faces the printhead - Thermal Transfer Printing: Verify that the printer is loaded with thermal transfer ribbon and that the ink-coated sided faces the receiving face material |
| Printing is faded | - Clean the printhead with the cleaning card - If printing is still faded, increase the density setup |
| Prints only partial label | - Print carriage not completely locked - Label caught on printhead. Remove and clean |
| Printer keeps printing or feeding when it should not | - Label caught on label gap sensor. Open top cover. Remove the label and clean using cleaning card or isopropyl alcohol - Possible firmware problem |
| Printing stops and indicator lamp lights orange | - Possible problem with label gap sensor. Perform auto adjust in the Test Mode - Possible media jam - Possible firmware problem |
| Label stuck on roller | - Open the peel-off mechanism and use fingers to peel away stuck label while manually rotating the roller. Do not use any sharp tools! Clean using a cleaning card or a cotton swab moistened with isopropyl alcohol |
| Label taken sensor does not hold printing until label/tag has been removed | - Label/tag bent down due to excessive length and/or too thin or soft media - Label/tag too short (less than 16 mm/.63 inches) - Label taken sensor or cable defective |
| Indicator lamp flashes red and the printing is interrupted | - Over temperature. Wait for the printer to cool down until the indicator lamp switches to green and – if the printing has been halted during a batch print job – the printing is automatically resumed |

# 12.	Specifications

Refer to the user guide PDF for the printer and media specifications

# 13.	Interfaces

13.1	Parallel Interface (EDP cable type AS1)

Handshake: DSTB to printer and BUSY to host

Printer connector: 36-pin female Centronics connector

13.2	Serial Interface (EDP cable type JO & JQ)

Printer connector: DB-9pin female RS-232 connector

13.3	1/. The combined load on the external +5V on both the parallel and the serial port must not exceed a total of 500 mA