# F. E. C. O.

| FECO: | BC#12 |
| --- | --- |
| PRODUCT: | Symbol CRD8800-400E 4 port Ethernet cradle |
| PRODUCT WEB SITE: | N/A |
| RELEASE DATE: |  |
| ESTIMATED MAN HOURS: | N/A |
| CLASS OF CHANGE: | INFORMATIONAL |
| REASON FOR CHANGE: |  |
| RELATED FECO/AB: |  |

Configuring the Symbol CRD8800-400E 4 port Ethernet cradle

Pre-requisites

Symbol 170013-000 cable (RJ45 (10pin) to 9 pin female)

Null modem cable (9 ping male to 9 Pin female)

Laptop / System with a 9 pin comport

Configuration

- The default configuration out of the box for the 4 port cradle is that it will connect to a DHCP server and obtain 6 IP addresses for its own use. This could be a problem if the DHCP scope available is limited, so we will configure a single IP address for the cradle with the remainder of the IP addresses being allocated from an internal range and translated (NAT) to the LAN IP address we give the cradle.

- Switch off the power to the Ethernet cradle if it is already powered on. Connect the 10pin RJ45 end of the Symbol cable to the expansion port on the cradle, connect the 9 pin male end of the null modem cable to the 9 pin female end of the Symbol cable and connect the 9 pin female end of the null modem cable to the laptop / system com port.

- Configure your terminal emulator (Anzio, Viaduct, hyperterm etc) to a baud rate of 115200, no parity, 8 bits and one stop bit (115200, 8, 1, N).

- Start up the terminal emulator and power on the cradle. Immediately keep pressing the ‘a’ button on the keyboard until the following menu appears (you need to press ‘a’ immediately after the power has been applied to the cradle. If the menu does not appear in 3-4 seconds power off and start again).

- Select option 1 ‘Public Network Settings and the following screen will be displayed

The IP address etc have already been set in this example to make it easier to see

what needs to be set.

The Static mode should be set to 1 to enable a static IP address (0 is DHCP).

The NAPT mode should be set to 1 to use only 1 IP address and NAT the remainder (not 6 addresses).

The cradle IP address should be set to the IP address that is required (192.168.96.6 in the example).

The router address / default gateway should be set if required (192.168.96.254 in the example).

The DNS server addresses can be set if the scanners need to resolve names to

addresses (195.60.21.1 and 195.60.21.2 in the example) and the subnet mask

should be set (255.255.255.0 in the example).

Once the options have been set enter ‘q’ to return to the previous menu and ‘F’ to

Save and reboot the cradle.

- Once the cradle has rebooted you can now use it with the Symbol PPT880 scanners with the eConnect software installed. These will then communicate directly on the network instead of via serial connection on a PC.

9 Pin Male to 9 Pin female Null Modem cable

9 Pin Male				9 Pin Female

10 Pin RJ45 to 9 Pin female (Symbol Cable 170013-000)

10 Pin RJ45			9 Pin Female

10 Pin RJ45 to 9Pin Female (complete cable)

10 Pin RJ45					     9 Pin Female