# F. E. C. O.

| FECO: | IO#14 |
| --- | --- |
| PRODUCT: | Perle IOLAN STS |
| PRODUCT WEB SITE: |  |
| RELEASE DATE: |  |
| ESTIMATED MAN HOURS: |  |
| CLASS OF CHANGE: | INFORMATIONAL |
| REASON FOR CHANGE: |  |
| RELATED FECO/AB: |  |

# Perle IOLAN STS

# 1.	Note

Throughout this FECO a SPACE in a string is shown by the symbol  and all other spaces are purely cosmetic. A carriage return is shown either by the symbol  or <CR>

# Introduction

The Perle IOLAN STS is a direct replacement for both the Perle / Specialix Jetstream and IOLAN products. It can be used with all of the EDP supported operating systems and software.

The IOLAN can be configured by means of an IOLAN style interface or by a Jetstream style interface and all options are available via both interfaces. The document will detail configuring the IOLAN via the IOLAN interface for BCT sites and via the Jetstream style interface for all other sites.

The IOLAN STS uses the standard IOLAN cables for serial connectivity and all the serial connections are located on the back of the terminal server.

# 3.	Prerequisites

A Laptop with a serial port (9 pin) and network capability

Cable Type LI (or Cable type JF to connect to the supplied adapter).

4.	Installation

The IOLAN STS comes in 2,4,8,16 and 32 port versions with the majority of them having a rack mounting facility. The IOLAN STS’s have the serial connection at the back of the unit, along with the network and power connections

The front of the STS shows the status of the server along with the (power, system ready) along with the network connection transfer rate and the network activity. The front panel also shows the status of the ports and has the connection to the admin / console port.

The power on / off switch is located on the front left of the unit and is easily accessible.

  - Connect the power lead to the power connection in the back of the IOLAN and then connect the

cable from the admin port on the front of the IOLAN STS to you laptop via the com port. On your laptop use HyperTerminal or Anzio version 15.1 to open a connection via the com port to the IOLAN. You should set the connection settings in the terminal emulator to be via the serial port with setting of 9600 baud, 8 data bits , 1 stop bit and Xon/Xoff flow control.  If you are using Anzio set the terminal emulation to be vt100. Now you can switch on the IOLAN using the power on / off switch on the front right of the IOLAN and you should get a Login: prompt displayed once the IOLAN has gone through its startup procedures.

  - The default administration login is ‘admin’ with a password of ‘superuser’. The first thing to set on the router is the admin password to do this enter :-

#### STS# set  user  .   password  password 

enter a password of iolan when prompted to and confirm it.

You now need to turn of the DHCP service for the iolan. To do this enter the following:-

STS# set  service  dhcp  off 

You now need to disable the guest login facility for the iolan :-

#### STS8# set  authentication  local  guest-mode  off 

  - For BCT sites we will use the IOLAN interface. To access this interface enter :-

STS#  iolan+

You will then have the Administration menu displayed.

Name: admin                ADMINISTRATION MENU             REMOTE-ADMIN


    gateway     Examine/modify gateway table.
    host        Examine/modify host table.
    line        Terminal configuration organised by line.
    password    Specify password to allow modification of menu items.
    port        Terminal configuration organised by port.
    quit        Returns user to command line interface.
    server      Examine/modify Server parameters.

4.3.1	To configure the server you need to use the arrow keys to select password and enter the password as iolan. Your menu will then change to incorporate additional options.

N.B. If you leave this screen idle, it will timeout and default back to the non administrative screen.

** Administrator **         ADMINISTRATION MENU              REMOTE-ADMIN


    access      Remote System Access (PPP).
    change      Change login and/or admin password.
    gateway     Examine/modify gateway table.
    host        Examine/modify host table.
    line        Terminal configuration organised by line.
    port        Terminal configuration organised by port.
    quit        Returns user to command line interface.
    server      Examine/modify Server parameters.
    trap        Examine/modify SNMP Trap parameters.

Select server and enter the following details: -

Name [ e.g. iolan1] – name of the iolan

IP address [ e.g. 192.1.1.23] – IP address for the iolan

Subnet mask [ e.g. 255.255.255.0] – subnet mash for the iolan

Boot host [ e.g. 192.1.1.21] – IP address of the host server

You cannot use an existing IOLAN configuration file to configure the IOLAN STS8 you have to create a new file and manually add the configuration for the ports.

When all the fields are entered correctly press ‘ESC’ and select ‘save and exit’.

4.3.2	Now you need to add entries in the Hosts table for the server or servers you wish to connect to. From the admin menu select host and add the names and IP addresses of the hosts:-

If you require a gateway to

4.3.3	Once you have updated the hosts press ‘Esc’ and select ‘save and exit’.

The Line screen shows details on all the ports on the server but the configuration is done through the port setup menu. Select port and enter the port number you wish to setup.

### VDU port setup

Select the correct speed, flow control, parity and stop bits.

Select the Name e.g. the alias name for the port [01]

Select the Terminal type [vt100]

Select TERM enter the terminal number to be used in esprit [e.g. 020]

Select Authentication [none]

Select the Access [Local]

Select Mode [Telnet]

Select Connection [Initiated]

Select Host [Enter the Hostname or IP address of the host system]

Select Remote port [23]

Select Local port [10001 for port 1, 10002 for port 2 etc]

Once you have made the amendments select Esc then save and exit.

Printer port setup

Select the correct speed, flow control, parity and stop bits.

Select the Name e.g. the name of the printer [e.g LP01]

Select the Terminal type [undef]

Select Authentication [none]

Select the Access [Remote]

Select Mode [Telnet]

Select Connection [None]

Select Host [Leave blank]

Select Remote port [23]

Select Local port [10001 for port 1, 10002 for port 2 etc]

Once you have made the amendments select Esc then save and exit.

This is the standard configuration for a printer using the iolan software to map the TCP/IP port and setup the printer. For a RCP printer the setup is as follows:-

Select the correct speed, flow control, parity and stop bits.

Select the Name e.g. the name of the printer [e.g LP01]

Select the Terminal type [undef]

Select Authentication [none]

Select the Access [Remote]

Select Mode [Raw]

Select Connection [Initiated]

Select Host [Leave blank]

Select Remote port [0]

Select Local port [514]

Once you have made the amendments select Esc then save and exit.

4.3.4	Once all the ports have been configured you should save the configuration on the device and to the application server. To do this from the menu select quit and from the prompt enter ‘l’ to logout of the iolan menu system. From the Login prompt enter admin as the user with a password of iolan.

e.g. 	Login		admin 

Password	iolan  

STS8 #	save 

STS8# 	netsave  configuration  system3  io1 

The example will save the configuration to a host that been setup called system3 and to a file on the host called ‘io1’. The file is located in the default /tftboot directory as long as tftp is enabled on the server.

Once you are sure the configuration file has been saved to the server, reboot the server by enter reboot from the command line

#### STS# reboot 

4.4	To configure the IOLAN STS using the Jetstream style interface (For Merchant and Charisma customers), login to the IOLAN using the admin user with a password of iolan. From the prompt enter screen or scr to bring up the configuration screen

    - From the main menu select server configuration and then server. You now need to enter the name of the IOLAN, the IP address and the subnet mask for the server.

Once you have entered the details press ‘press ‘’ and select ‘Accept and exit form’ and press ‘Esc’ to get back to the main menu.

4.4.2	From the main menu select ‘Network configuration’ and then ‘host table’. Select ‘add host’ and add alias names for both the server you wish to connect to and the default gateway (if applicable). You now need to select change host and change the IP addresses for the two aliases you have just entered from 0.0.0.1 to the correct IP addresses. Once you have made the amendments, press ‘’ and select ‘Accept and exit form’.

    - If you need to add a default gateway, select ‘Gateways’ and then ‘add gateway’ and select the name you have entered as a host name for the default gateway. Select ‘Change gateway’ and change the IP Address for the default gateway to 0.0.0.0 and the service to default.

Once you have entered the details press ‘’ and select ‘Accept and exit form’. Now press ‘Esc’ twice to get back to the main menu.

    - From the main menu select ‘Time configuration’ and ‘set time’. Enter the correct date and time and press ‘’ and select ‘Accept and exit form’.

4.4.4	The basic server configuration is now done and you now  need to configure the ports to either be printers or VDU’s (Trueport or Telnet). To do this select ‘Line configuration’ from the main menu, then ‘Line port settings’ and the line number you wish to configure.

4.4.5	For RCP printers setup the port with the following :-

The line name should reflect the name of the printer e.g. LP01. The service should be set for printer and the interface should be EIA-232. You now need to select settings and set the correct baud rate etc and set the flow to soft.

Once you have made the amendments press ‘’ and select ‘exit’.  Press ‘’  again and select ‘save and exit’ to exit from the port screen.

    - For a standard VDU/serial connection:-

Set the Line name to be whatever the port number is unless there is another naming convention available i.e. the name of the user. Set the Service to Dir Tel and select settings

You should select the Host from the list of hosts you have entered in the hosts table previously and select the Termtype you wish to use.

Once you have amended the Host and the termtype (if required) press ‘’ and select ‘exit’.

Press ‘’  again and select ‘save and exit’ to exit from the port screen.

    - For Trueport connections:-

Set the service to ‘Sil Raw’ and select settings. From the settings box enter the host the port should connect to from the hosts table and the trueport port that the port will connect to e.g. 10003 for trueport X0003. Once you have amended this press ‘’ and select ‘exit’. You can now configure the baud rate by selecting the settings next to the serial interface and configuring the baud rate you require etc. Once you have amended this press ‘’ and select ‘exit’. From the Line screen press ‘’  again and select ‘save and exit’ to exit from the menu.

    - Once you have configured all the ports required you need to save the settings. To do this get back to the main menu screen and select ‘command line mode’ and then ‘save and exit’.. You can now save the configuration using the save command and reboot the server

STS8 #	save 

#### STS# reboot 

Once the server has rebooted the connections should become active and they can then be tested.