# F. E. C. O.

| FECO: | FECO IO#16 |
| --- | --- |
| PRODUCT: | Axel Platine M75c |
| PRODUCT WEB SITE: |  |
| RELEASE DATE: |  |
| ESTIMATED MAN HOURS: |  |
| CLASS OF CHANGE: | INFORMATIONAL |
| REASON FOR CHANGE: | New Product |
| RELATED FECO/AB: |  |

# Configuring the Axel Platine M75 on Esprit systems

# 1.	Note

Throughout this FECO a SPACE in a string is shown by the symbol  and all other spaces are purely cosmetic. A carriage return is shown either by the symbol  or <CR>

# Introduction

The Axel Platine M75c differs from the standard M75 in that you have the ability to configure all of the keys on the keyboard to a different value.  This negates the requirement for a specifically altered firmware for the BCT customers as the function key values can be manually setup.

The print screen, scroll lock and pause break keys should be re-mapped to be F13, F14 and F15 and when the shift key is depressed at the same time as the print screen, scroll lock and pause break keys they will pass a different string to the software.

The Axel platines have 2 modes of Operation TCP/IP (on the network) or Serial (Via a terminal server).

# 3.	Prerequisites

Axel Platine M75c

Monitor

Keyboard

Mouse

Relevant LAN / Serial cables

4.	Installation

  - Basic Installation

The Axel Platine M75 is two thirds of the size of the previous M65 version. It is 1.35 KG in weight and has an operating temperature range between 5 and 35C.

The M75 has 2 RJ45 AUX ports and no 25-pin Aux connection. The power cable is non-standard and therefore is not inter changeable between different models.

The back of the M75 is shown above and as you can see it has a number of connections available.

To start the M75 connect the power lead to the power connector (11), connect the VGA cable from the monitor (1), connect the keyboard and mouse (2 & 3), connect the LAN cable (if running TCP/IP (7)) and connect any other serial / parallel connections required (Aux1/2 and the parallel port).

Once all the connections are connected switch the Platine on via the On/Off switch (10), the Platine will ‘Beep’ and the monitor will display the startup screens.

Once the Axel has started up you will have the Quick setup screen displayed.

Change the keyboard from American to English by pressing the spacebar to select the keyboard and selecting English from the list and pressing ‘’ to confirm the setting.

Change the emulation from ANSI to WYSE 50/60/120.

On the network setting, if the Axel Platine is to get its address via DHCP you need not add an IP address for the Axel Platine. If you need to add a static IP address then you need to change ‘Enable DHCP’ to ‘no’ and add the IP address the Axel should have and the IP address of the default router.

You can also set the name for the Terminal and the host name and IP address of the system.

Once you have added these entries select ‘VALID’ and the system will then reboot.

4.2	TCP/IP Installation

To access the Axel Platines configuration screen you need to press control, alt and escape together on the keyboard (Ctrl-Alt-Esc).  Select TCP/IP and Interface and check that the IP address, netmask and terminal name are correct. If they are not, amend them and select ‘VALID’.

If you have configured a default route on the Quick setup screen it should be displayed in the ‘Routers’ screen. If it is not or it is incorrect replace the default route with the correct value or add a separate route if required. Select ‘VALID’ once you have made the amendments.

The last network information required is a list of hosts. Select the ‘Host’ screen and add the name of the host you wish to connect to and the IP address of the host. Once you have entered these select ‘VALID’.

    - Get back to the main menu options by pressing escape (Esc) and select Terminal and then Session 1. For Esprit customers the emulation needs to be Wyse 50/60/120, the protocol should be telnet and change the host to be the hostname you have added to the hosts table. Select connection properties and change the TERM Value to be the line number for the process in Esprit e.g.020 (this can be obtained from the user or Esprit support). Select ‘VALID’ once you have entered the value required.

    - You now need to configure the specific function keys / string values for the Esprit software. To do this select User Defined Sequences, select sequence #1 (press spacebar), press the spacebar at the keys sequence screen and then you will be prompted to press the key(s) sequence required. Press the Print Screen key and <PrntScr> will be displayed in the key sequence. Enter an associated string of \01L\0D and select valid.

    - You now need to repeat the process for the scroll lock and pause break keys entering the following values :-

\01M\0D		Scroll Lock

\01N\0D		Pause Break

    - You now need to setup the 3 required string values for the shifted options for the same keys i.e. shift F13,F14,F15 in the application. The metod for this is exactly the same with the exception that the shift key is depressed before the print screen, scroll lock or pause break key in the recorded key sequence to get a sequence of 2 keys rather than only one. The values required are:-

Shift + Print Screen			\01l\0D		(l is a lowercase L)

Shift + Scroll Lock			\01m\0D

Shift + Pause Break			\01n\0D

Once all the amendments have been made you should get a list of 6 defined sequences displayed :-

#1	<PrntScr>		:	“\01L\0D”

#2	<ScrlLock>		:	“\01L\0D”

#3	<Pause>		:	“\01L\0D”

#4	<Shift><PrntScrn>	;	“\01l\0D”

#5	<Shift><ScrlLock>	;	“\01m\0D”

#6	<Shift><Pause>	;	“\01n\0D”

Once you have checked the values for the sequences select valid to save the settings.

    - Leave the remainder of the settings unchanged and select ‘VALID’ and press escape (Esc) to get back to the main configuration menu.

    - To configure any of the three ports on the Axel (parallel, Aux1and Aux2) to be printers,

select Ports, Auxiliary Ports and then the port you wish to configure e.g. Parallel.  Change the service to be ‘lpd’ , select parameters and change the Printer port name to be the name of the printer e.g. LP01. Once you have entered the details select ‘VALID’ and ‘VALID’ again to get back to the Auxiliary ports menu. The configuration is slightly different for the Aux1 and Aux2 ports but only in that you setup the baud rate, flow control etc for the printer’s serial connection.

The last piece of configuration required is to turn of additional functionality for the parallel port. To do this from the main menu select Advanced and then tunings and ensure the Parallel port has ‘Supplemental Signals to Handle’ set to none and ‘Operating Mode’ set to standard. If they are not set correctly amend them and select ‘VALID’ once you have made the amendments. Press escape twice to get back to the main menu and select Exit. You will be advised the settings have been changed and that you need to reboot, select ‘YES’ to reboot and the system will reboot.

    - Once the system has rebooted you can press the Alt and F1 keys to login to session 1 which should get you to a login prompt on the Esprit server.

4.3	Serial Installation

    - For the serial setup we will assume that a standard IOLAN / Jetstream setup is already in place

and that the Axel is a replacement for an existing Wyse 55 screen. You will need a cable type ‘LJ’ which is a RJ45 connection to the Aux port on the Axel and a 25 pin male connection to plug into the existing cabling (BCT038 etc). Connect the cable from the Axel to the existing cabling and press (Ctrl-Alt-Esc) to get into the Axel setup screen.

    - From the main menu select Terminal and then session 1. Set the emulation to be WYSE50/60/120 and the protocol to be aux1 or aux2 (depending on what port the cable is connected to). You need not amend any of the other parameters and select ‘VALID’ to save the configuration and press ‘Esc’ to return to the main menu.

    - You now need to configure the baud rate, stop bits etc for the connection.  Select Ports, Auxiliary ports and then the port you wish to configure i.e. aux1 or aux2. You will see the Operating Mode is set to Serial session. You now need to ensure the speed, format and handshaking parameters are correct for the connection to the terminal server and select ‘VALID’ once they are correct. Press ‘Esc’ to get back to the main menu and select ‘Exit’.

    - Once the system has rebooted you can press the Alt and F1 keys to login to session 1 which should get you to a login prompt on the Esprit server.

5	Using the Axel Platine M75 for POS (replacing a Wyse 55)

# 5.1	Existing setup

The existing POS configuration for BCT customers involves using a Wyse 55 ESP terminal with a cash drawer connected to it via the parallel port or with the cash drawer shared via a Belkin parallel switch. The POS configuration can also have a serial scanner attached via a ‘y’ cable to the serial input from the terminal server.

# 5.2	New Setup

The new configuration for BCT customers involves using an Axel Platine M75 in the place of the Wyse 55 screen. The Axel Platine can be connected directly to the network via a standard RJ45 network connection or serially via one of the Aux ports to an Iolan or Jetstream.

If the cash drawer needs to be shared between more than one screen, then a Belkin parallel port switch can be used to connect to the parallel ports on the VDU’s or Axel Platines.

The diagram shows the Axel M75 connected on the network (TCP/IP), with the cash drawer connected to the parallel port on the M75 and the parallel input on the cash till. The M75 is configured to the standard TCP/IP setup as detailed in section 4.2 and requires no additional configuration. If the cash drawer is to be shared between multiple Axel M75’s or Wyse 55 screens then from the cash drawer connect a 25 pin straight through cable (male to female) to the common output on the Belkin parallel switch to the parallel input on the cash drawer. Then connect a 25 pin straight through (male to male) from the M75’s parallel port to one of the 4 input ports on the Belkin switch. It now functions in exactly the same way as a single cash drawer attached directly to the Axel M75.

  - Serial POS setup

If an Axel Platine is to be used in place of a Wyse 55 screen without any other configuration changes being made, (i.e. If the Axel is not being connected directly to the network but to a terminal server) then you can setup one of the Aux ports on the Axel to act as a serial port and connect directly to the terminal server (Section 4.3) and the existing connection to the cash drawer (direct or via a Belkin switch) can connect directly into the parallel port.