### F. E. C. O.

| FECO: | Open8# |
| --- | --- |
| PRODUCT: | RSH printing to Axel Products |
| PRODUCT WEB SITE: |  |
| RELEASE DATE: | 3rd May 2005 |
| ESTIMATED MAN HOURS: |  |
| CLASS OF CHANGE: | INFORMATIONAL |
| REASON FOR CHANGE: |  |
| RELATED FECO/AB: |  |

# RSH printing to Axel Products

Section	Page

1.	Note	1

2.	Introduction	1

3.	Prerequisites	1

4.	Configuring the Hardware	2

5.	Configuring an Axel Netterm	3

6.	Configuring the Operating system	3

7.	Adding the printers into UniVision	5

  - Note

Throughout this document a carriage return is shown either by the symbol  or <CR>, a SPACE in a string is shown by the symbol  all other spaces are purely cosmetic. A TAB is shown as <Tab>, Function keys as <F10> and control as <Ctrl>

  - Introduction

Remote shell (RSH) can be used to print to Axel terminal servers and Netterms. This can be very usefull as the standard method of BSD (LPD) printing to these devices can cause a problem on the Open Unix 8 / Unixware operating systems. The setup for this style of printing is very similar to that of rcp printing and should therefore be very easy to implement.

This document details configuring RSH printers for Unix and UniVision. It does not deal with setting up terminals on the Axel products only printers

  - Prerequisites

    - Open Unix 8 / Unixware 7.x.x operating system installed

    - Port 514 is open on any firewall between the main system and the netterm / terminal server

    - You are able to amend the configuration of the terminal server / netterm

  - Configuring Axel Terminal servers (4010 / 4020)

4.1	Connect the VDU to the config port (CH. 4 on a 4010 or CH.8 on 4020) using cable type KN and set the switch on the back of the Office Server to conf. Make sure the VDU is set to 9600 8 1 None VT100. Power on the Office Server

Select option 1-NETWORK SETUP, then option 2-IP ADDRESS and <CR> to proceed

Enter the name of the server e.g. axel1 <CR> Enter the IP address e.g.192.1.1.3 <CR> and then the relevant IP mask (subnet mask) e.g. 255.255.255.0 <CR> Enter Q to finish and return to the main menu

4.2	Select option 1-NETWORK SETUP, option 4-HOSTS then option 1-Host Add and <CR> to proceed

4.3	Enter the host IP address e.g. 192.1.1.2 <CR> and then the host name e.g. linux1 <CR> to finish and return to the previous menu enter q <CR>

4.4	If routes are required they can also be setup from this menu. If no other network configuration is required enter Q <CR> to Return to the main menu

4.5	To setup a port select option 2-SERIAL LINE SETUP and then select the line you wish to setup 1-4 or 1-8 depending on the Office Server. Select option 1-Line Parameters and setup with the following parameters for a printer. The baud rate MUST be the same as the printers baud rate

LINE TYPE	:direct

BAUD RATE	:9600

DATA FORMAT	:8/1/none

TX FIFO SIZE	:12

INPUT FLOW CNTL	:xon

FLOW CNTL CHARS PASSTHRU	:y

OUTPUT FLOW CNTL	:xon

PERIPHERAL PRESENCE TYPE	:none

4.6	For a Printer, setup the Network Service with the following parameters: -

SERVICE MODE	:server

SERVICE TYPE	:lpdrcmd

LPD/RCMD QUEUE NAME	:e.g.UVpt1

LPD BEFORE PRINT SEQ.	:

LPD AFTER PRINT SEQ.	:\0c

LPD NL TO CRNL CONV.	:n

TCP KEEPALIVE TIMEOUT	:0

The LPD after print sequence needs to be set to enable a form feed at the end of the job.

4.7	Once the server is configured an entry should be put in the /etc/hosts file with the IP address

and the alias for the terminal server.

E.g.	192.168.1.0	ax4020

  - Configuring an Axel Netterm

5.1	To configure the netterm press <Ctrl> <Alt> <Esc>. The setup menu will be displayed. Select TCP/IP <CR> and then Interface <CR>, use the spacebar to toggle DHCP to No and enter the IP address and the Net mask required for the netterm, select Valid <CR>

5.2	If the Netterm is on a WAN, select Routers <CR> and enter the IP address of the Default Router. Select Valid <CR>

5.3	Select Hosts <CR> and enter the host name and the IP address of the host you need to connect to. Select Valid <CR> then <Esc>

5.4	Select Terminal <CR>, General <CR> and set the Screen Type to CRT Standard Monitor and the Keyboard type to English. Select Valid <CR>

5.5	To configure the printers select Aux ports <CR>, and select the port you wish to configure <CR>. Select the associated service <CR> and select rcmd (remote command) <CR>. Then select the net service parameter (name of the queue for the printer) and enter the name for the print queue <CR>. For printers connected to the Aux or parallel ports set Auto Form Feed to Yes OR, where this option is not present, enter \0C (backslash, zero, C) in the Post-print String (This is firmware dependant and only available when the port service is set, e.g. lpd, and the keyboard is set to English). Select Valid <CR> twice and <Esc> back to the main menu

5.6	Select Advanced <CR>, tunings <CR> and then change TCP/IP assignment to FIXED <CR> and select Valid <CR>

5.7	<Esc> to get back to the main menu and select Exit <CR> select YES to save the new settings. The netterm will save and update the settings

  - Configuring the Operating system

6.1	There are 3 stages to adding an rsh printer on Open UNIX 8 / Unixware 7.x.x - interface script creation, device file configuration and the adding of printers to the system

6.2	Interface script

A modified interface script must be created for each printer that is to be added. The scripts are located in the /usr/lib/lp/model directory, where you will find the default UVstandard script

cd  /usr/lib/lp/model 

Copy the original UVstandard script to UVstandard.orig

cp  UVstandard  UVstandard.orig 

Copy UVstandard to UVstandardx where x is a unique file number not already in use

cp  UVstandard  UVstandardx 

6.3	Amend the new file created, UVstandardx, and replace the line that reads:-

0<$ {file}  eval  ${FILTER}  2>&1  1>&3

With: -

rsh  { alias}  {queue name}  <  ${file}  2>&1  1>&3

E.g.	rsh  ax4020  UVpt1  <  ${file}  2>&1  1>&3

This will send the print job to the terminal server with the alias of ax4020 and to the port with the queue name of UVpt1.

6.4	Device file

Each printer must have its own device file. They need to be created with the attributes of a null device and individually named. First find the major minor numbers of the null device on the system. Enter the following command: -

ls  –l  /dev/null 

You will get an output similar to the one below: -

crw-rw-rw-	1	root	sys	39,	0   Oct 30 17:06    /dev/null

6.5	Here the major and minor numbers are 39 and 0. Create an individual file for your printer with its unique file number. The command is as follows: -

mknod  /dev/znullx  c  39  0 	(Where x is the unique file number)

Change the permissions on the file so every user can access it: -

chmod  666  /dev/znullx		(Where x is the unique file number)

6.6	Adding the printers to the system

The addition of the printers to the system is virtually the same as adding local printers but the devices and interface scripts differ. The syntax is: -

lpadmin  -p UVpt1  –v  /dev/znullx  -m  UVstandardx  - T  UVdumb  –o  nobanner 

Where UVLP01 is the printer name, /dev/znullx is the unique device file, UVstandardx is the edited interface script with the details for the terminal server and port, UVdumb is the printer type and there is no banner

6.7	Enable and accept the printers: -

enable  UVpt1 

accept  UVpt1 

6.8	Test the printer by sending a print from the command line: -

lp  –d  {printer name}  {file} 

E.g.	lp  –d  UVpt1 /etc/hosts 

This will send the /etc/hosts file to the printer UVLP01. Once the printer has been tested it can be added into the relevant application and tested from there

  - Adding the printers into UniVision

7.1	Login to UniVision on the console and <CR> to keep the e console type as its default value and from the menu select option (1) UVsetup

7.2	If there are other users on the machine it will prompt you to enter (y / n): - to continue, enter y <CR>

7.3	Enter 5 Maintain Printer Records and then enter A to add a printer and enter the Linux name for the printer e.g. UVptr1. You will be advised the printer is added on process ‘xx’. Make a note of the process number the printer has been added on

7.4	Enter E to exit and 0 to exit from UVsetup

7.5	Select ‘Enter UniVision’, logon to the SYSPROG account and press escape to get to UCL

7.6	Start the printer on a specific forms queue: -

E.g.	STARTPR  0 , 0 , 0 , S1 

This will start printer number 0, on forms queue 0, with no page skip, on process 1

7.7	To test the printer assign it to a forms queue: -

SP-ASSIGN  F0 	(this will assign you to forms queue 0)

Then send a print job to the printer: -

LLC  (P 